|  | D-Link DGS-3100-xx |  |
| --- | --- | --- |
|  | Release Notes |  |
|  | 1.04 June 2007 Not approved by Document Control. For review only. |  |

| Document Status |  |  |
| --- | --- | --- |
| Advanced Information | This document contains design specifications for initial product development. Specifications may change without notice. Contact Marvell Field Application Engineers for more information. |  |
| Preliminary Information | This document contains preliminary data, and a revision of this document will be published at a later date. Specifications may change without notice. Contact Marvell Field Application Engineers for more information. |  |
| Final Information | This document contains specifications on a product that is in final release. Specifications may change without notice. Contact Marvell Field Application Engineers for more information. |  |
| Revision Code: |  |  |
| Preliminary: |  | Technical Publication: |

| Document Conventions |  |
| --- | --- |
|  | Note |
|  | Provides related information or information of special importance. |
|  | Caution |
|  | Indicates potential damage to hardware or software, or loss of data. |
|  | Warning |
|  | Indicates a risk of personal injury. |

Disclaimer

This document provides preliminary information about the products described, and such information should not be used for purpose of final design. Visit the Marvell® web site at www.marvell.com for the latest information on Marvell products.

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose, without the express written permission of Marvell. Marvell retains the right to make changes to this document at any time, without notice. Marvell makes no warranty of any kind, expressed or implied, with regard to any information contained in this document, including, but not limited to, the implied warranties of merchantability or fitness for any particular purpose. Further, Marvell does not warrant the accuracy or completeness of the information, text, graphics, or other items contained within this document. Marvell makes no commitment either to update or to keep current the information contained in this document. Marvell products are not designed for use in life-support equipment or applications that would cause a life-threatening situation if any such products failed. Do not use Marvell products in these types of equipment or applications. The user should contact Marvell to obtain the latest specifications before finalizing a product design. Marvell assumes no responsibility, either for use of these products or for any infringements of patents and trademarks, or other rights of third parties resulting from its use. No license is granted under any patents, patent rights, or trademarks of Marvell.These products may include one or more optional functions. The user has the choice of implementing any particular optional function. Should the user choose to implement any of these optional functions, it is possible that the use could be subject to third party intellectual property rights. Marvell recommends that the user investigate whether third party intellectual property rights are relevant to the intended use of these products and obtain licenses as appropriate under relevant intellectual property rights.

Marvell comprises Marvell Technology Group Ltd. (MTGL) and its subsidiaries, Marvell International Ltd. (MIL), Marvell Semiconductor, Inc. (MSI), Marvell Asia Pte Ltd. (MAPL), Marvell Japan K.K. (MJKK), Marvell Semiconductor Israel Ltd. (MSIL), SysKonnect GmbH, and Radlan Computer Communications, Ltd.

Export Controls.  With respect to any of Marvell’s Information, the user or recipient, in the absence of appropriate U.S. government authorization, agrees: 1) not to re-export or release any such information consisting of technology, software or source code controlled for national security reasons by the U.S. Export Control Regulations ("EAR"), to a national of EAR Country Groups D:1 or E:2; 2) not to export the direct product of such technology or such software, to EAR Country Groups D:1 or E:2, if such technology or software and direct products thereof are controlled for national security reasons by the EAR; and, 3) in the case of technology controlled for national security reasons under the EAR where the direct product of the technology is a complete plant or component of a plant, not to export to EAR Country Groups D:1 or E:2 the direct product of the plant or major component thereof, if such direct product is controlled for national security reasons by the EAR, or is subject to controls under the U.S. Munitions List  ("USML").  At all times hereunder, the recipient of any such information agrees that they shall be deemed to have manually signed this document in connection with their receipt of any such information.

Copyright © 2004.  Marvell.  All rights reserved.  Marvell, the Marvell logo, Moving Forward Faster, Alaska, Prestera and GalNet are registered trademarks of Marvell.  Discovery, Fastwriter, GalTis, Horizon, Libertas, Link Street, NetGX, PHY Advantage, Raising The Technology Bar, UniMAC, Virtual Cable Tester, and Yukon are trademarks of Marvell.  All other trademarks are the property of their respective owners.

Table of Contents

Section 1 Introduction	3

1.1	Goal of this Document	3

1.2	Platforms	3

1.3	User Documentation Specifications	3

1.4	System Specifications	3

1.4.1	System Firmware Version Details	3

Section 2 Supported Firmware Functionality	3

2.1	Feature Overview	3

2.1.1	General Features	3

2.1.2	MAC Address Supported Features	3

2.1.3	Layer 2 Features	3

2.1.4	VLAN Supported Features	3

2.1.5	Link Aggregation	3

2.1.6	Quality of Service Features	3

2.1.7	Device Management Features	3

2.1.8	Security Features	3

Section 3 Issues Resolved in this Release	3

Section 4 Known Restrictions and Limitations	3

Appendix A	Revision History	3

List of Tables

Table 1: Layer 2 Features	3

Table 2 Quality of Service	3

Table 3: Web Based Interface	3

Table 4: CLI Commands	3

Table 5: Management & System Features	3

Table 6: Revision History	3

Introduction

Goal of this Document

The purpose of this document is to detail the implemented and not-fully-implemented features in the DGS-3100-xx code.

The document is divided into the following sections:

Section 1 – Introduction

Section 2 – List of features that are/will be available at product release

Section 3 – List of issues that have been resolved in this software release

Section 4 – List of known issues on this software release. The list includes non-supported features in this software release.

Platforms

This document provides information for the specific version of the following platform:

- DGS-3100-24 - 24-port 10/100/1000Base-T L2 Stackable Management Switch with 4 Combo SFP

- DGS-3100-24P - 24-port 10/100/1000Base-T PoE L2 Stackable Management Switch with 4 Combo SFP

- DGS-3100-48 - 48-port 10/100/1000Base-T L2 Stackable Management Switch with 4 Combo SFP

- DGS-3100-48P - 48-port 10/100/1000Base-T PoE L2 Stackable Management Switch with 4 Combo SFP

User Documentation Specifications

The following user documents are available for these devices:

D-Link DGS-3100 Series CLI Guide.

D-Link DGS-3100 series H/W Installation Guide

D-Link DGS-3100 series WEB System User Guide

System Specifications

System Firmware Version Details

| Name of the Boot Code Image | Version No. | Release Date |
| --- | --- | --- |
| dgs_31xx_boot-10003.rfb | 1.0.0.3 |  |

| Name of the Main Software Application Program Image | Version No. | Release Date |
| --- | --- | --- |
| dgs_31xx-10036.ros | 1.00.36 | May 31, 2007 |

Supported Firmware Functionality

Feature Overview

This section describes the device features. For the details regarding the system functionalities, please refer to the User's Guide.

General Features

Head of Line Blocking

Head of Line (HOL) blocking results in traffic delays and frame loss caused by traffic competing for the same egress port resources. HOL blocking queues packets, and the packets at the head of the queue are discarded before packets at the end of the queue.

Flow Control Support (IEEE 802.3X)

Flow control enables lower speed devices to communicate with higher speed devices, by requesting that the higher speed device refrains from sending packets. Transmissions are temporarily halted to prevent buffer overflows.

Note that back pressure and flow control cannot work together.

Back Pressure Support

On half-duplex links, the receiving port prevents buffer overflows by occupying the link so that it is 
unavailable for additional traffic.

Note that back pressure and flow control cannot work together.

MDI/MDIX Support

The device automatically detects whether the cable connected to an RJ-45 port is crossed or straight through. Standard wiring for end stations is Media-Dependent Interface (MDI) and the standard wiring for hubs and switches is known as Media-Dependent Interface with Crossover (MDIX).

Auto Negotiation

Auto negotiation allows the device to advertise modes of operation. The auto negotiation function provides the means to exchange information between two devices that share a point-to-point link segment, and to automatically configure both devices to take maximum advantage of their transmission capabilities.

Auto-negotiation advertisement is also supported. Port advertisement allows the system administrator to configure the port speed and duplex advertised.

MAC Address Supported Features

MAC Address Capacity Support

The device supports up to 8K MAC addresses. The device reserves specific MAC addresses for system use.

Static MAC Entries

MAC entries can be manually entered in the Bridging Table, as an alternative to learning them from incoming frames. These user-defined entries can be configured not to be subject to aging, and to be preserved across resets and reboots.

Self-Learning MAC Addresses

The device enables automatic MAC address learning from incoming packets. The MAC addresses are stored in the Bridging Table.

Automatic Aging for MAC Addresses

MAC addresses from which no traffic is received for a given period are aged out. This prevents the Bridging Table from overflowing.

Layer 2 Features

Port Mirroring

Port mirroring monitors and mirrors network traffic by forwarding copies of incoming and outgoing packets from a monitored port to a monitoring port. Users specify which target port receives copies of rx, tx or all traffic passing through a specified source port.

Broadcast Storm Control

Storm Control enables limiting the amount of Multicast and Broadcast frames accepted and forwarded by the device. When Layer 2 frames are forwarded, Broadcast and Multicast frames are flooded to all ports on the relevant VLAN. This occupies bandwidth, and loads all nodes connected on those ports.

VLAN Supported Features

VLAN Support

VLANs are collections of switching ports that comprise a single broadcast domain. Packets are classified as belonging to a VLAN based on either the VLAN tag or based on a combination of the ingress port and packet contents. Packets sharing common attributes can be grouped in the same VLAN.

GVRP Support

GVRP enables the switch to dynamically create VLANs on links with other devices running GVRP. It enables the switch to automatically create VLAN links between GVRP-aware devices. This reduces configuration efforts and minimizes the chances for errors in VLAN configuration.

Spanning Tree Protocol (STP, RSTP and MSTP)

802.1d Spanning tree, 802.1r Rapid Spanning Tree and Multiple Spanning Tree protocols are supported to allow bridges to automatically prevent and resolve L2 forwarding loops. Switches exchange configuration messages using specifically formatted frames and selectively enable and disable forwarding on ports.

Link Aggregation

Link Aggregation

Up to eight Aggregated Links may be defined, each with up to eight member ports, to form a single Link Aggregated Group (LAG). This enables:

Fault tolerance protection from physical link disruption

Higher bandwidth connections

Improved bandwidth granularity

High bandwidth server connectivity LAG is composed of ports with the same speed, operating at full-duplex.

Quality of Service Features

Class Of Service 802.1p Support

The IEEE 802.1p signaling technique is an OSI Layer 2 standard for marking and prioritizing network traffic at the data link/MAC sub-layer. 802.1p traffic is classified and sent to the destination. No bandwidth reservations or limits are established or enforced. 802.1p is a spin-off of the 802.1Q (VLANs) standard. 802.1p establishes eight levels of priority, similar to the IP Precedence IP Header bit-field.

Quality of Service Support

To overcome unpredictable network traffic and optimize performance, you can apply Quality of Service (QoS) throughout the network to ensure that network traffic is prioritized according to specific criteria. The switch supports two modes of QoS: basic and advanced.

Device Management Features

DHCP Clients

DHCP allow dynamic allocation of an IP address.

SNMP Versions 1, 2 and 3

Simple Network Management Protocol (SNMP) over the UDP/IP protocol controls access to the system, a list of community entries is defined, each of which consists of a community string and its access privileges. There are 3 levels of SNMP security read-only, read-write and super. Only a super user can access the community table.

Web Based Management

With web based management, the system can be managed from any web browser. The system contains an Embedded Web Server (EWS), which serves HTML pages, through which the system can be monitored and configured. The system internally converts web-based input into configuration commands, MIB variable settings and other management-related settings.

Configuration File Download

The device configuration is stored in a configuration file. The Configuration file includes both system wide and port specific device configuration. The system can display configuration files in the form of a collection of CLI commands, which are stored and manipulated as text files.

TFTP Trivial File Transfer Protocol

The device supports software and configuration upload/download via TFTP.

Remote Monitoring

Remote Monitoring (RMON) is an extension to SNMP, which provides comprehensive network traffic monitoring capabilities (as opposed to SNMP which allows network device management and monitoring). RMON is a standard MIB that defines current and historical MAC-layer statistics and control objects, allowing real-time information to be captured across the entire network.

Syslog

Syslog is a protocol that enables event notifications to be sent to a set of remote servers, where they can be stored, examined and acted upon. The system sends notifications of significant events in real time, and keeps a record of these events for after-the-fact usage.

Security Features

Port Based Authentication (802.1x)

Port based authentication enables authenticating system users on a per-port basis via an external server. Only authenticated and approved system users can transmit and receive data. Ports are authenticated via the Remote Authentication Dial In User Service (RADIUS) server using the Extensible Authentication Protocol (EAP).

RADIUS Client

RADIUS is a client/server-based protocol. A RADIUS server maintains a user database, which contains per-user authentication information, such as user name, password and accounting information.

ACLs support

The device supports ACLs (Access Control Lists), the ACL can check fields in the packet and activate the required action in case of a match. The action can be permit, deny and QoS operation.

Issues Resolved in this Release

| Spanning Tree | If a BPDU is sent to an edge port with a priority lower than the DUT (assuming the DUT is the root bridge), then the link partner (connected through the edge port) becomes the root bridge ***according to the DUT as well***. The port is still treated as portfast (or edge port in DGS terminology) in the convergence process. (i.e. It is moved automatically to the forwarding state without going through listening/learning). | 83705 |
| --- | --- | --- |
| PoE ports | The web shows the PoE ports for the first unit only.. Workaround: It can be checked through the CLI | 85294 |
| SNMPv3 | It’s possible to create a SNMPv3 user with a group name that wasn’t created and defined yet via the web Workaround: Correct configuration will work properly. | 84493 |
| SNMPv3 | It’s possible to create a SNMPv3 group with a view name that wasn’t created by the user or already exist at the view table Workaround: Correct configuration will work properly. | 84494 |
| SNMPv3 | It’s possible to create a SNMP community with a view name that wasn’t created by the user or already exist at the view table Workaround: Correct configuration will work properly. | 84496 |
| ACLs | wrong behavior in the WEB when configuring access_profile - if the user create an access profile with a wildcard of "0"s (and any destination) when trying to add an access id you will get an error message , in the CLI the access id will not appear as it should. when configuring the same thing via CLI there is no problem this problem causes problem when trying to download the configuration file to the device Workaround: This scenario can be configured through the CLI | 85345 |
| CLI: SNMP | - Using CLI it’s possible to create SNMP community/groups/users with none existing “view”. - Workaround: The user should define community/group/users with existing view. | 81351 |
| CLI: config access_profile | - It is impossible to choose auto_assign option in "config access_profile " command) - Workaround: The user can work without auto assign or configure through the web | 83685 |
| CLI: | - In the cli, when try to define more then 2 methods for login/enable the command not completed when use "tab" or "?" - Workaround: The command is working if the user writes it properly. | 84052 |
| CLI: PVE (Traffic Segmentation) | - In the cli, it is impossible to configure PVE with LAG uplink - Workaround: Can be configured using the WEB GUI | 84231 |
| CLI: config stp | - when typing the command "config stp xyxyxy m" and pressing "enter" - the device gets stuck - Workaround: If the user inserting the correct command the device is working fine. | 84560 |
| CLI: config access_profile | - when using the ACL commands in specific way, a fatal error occurs when trying to upload the running config. - Workaround: The commands are working, the config file can’t be used | 85186 |
| CLI: config ports… | When typing: config ports 1 learning disable s **Enter** The device stucks, and doesn't respond . - Workaround: If the user type legal syntax the device is working fine. | 85337 |
| Configuration file | In some cases it is impossible to copy configuration file to the device Workaround: It happens with some specific parameters (like safeguard). In this case the user will need to configure it manually on the target system. | 85140 |
| Safeguard | Safeguard is not functioning on units in stack that not the master. | 85167 |
| SNMP User | snmp users are not written in the running configuration Workaround: Uploading the configuration will require reconfiguration of the SNMP users. | 84699 |
| Startup Config | the device doesn't delete all the startup config in reset Workaround: Link Aggregation configuration that remains can be deleted manually . | 85132 |
| Configuration with certificate 2 | Downloading configuration, that sets certificate 2 as active - doesn't configured it at startup Workaround: the command should be repeated after download of the configuration file. | 85287 |
| IP Address | After downloading conf. without specifying ip (or dhcp) - device starting up with ip 1.0.0.0 Workaround: the user can set the correct IP after the download | 85287 |
| CLI: show VLAN | - There should be a "tagged ports" line in the "show vlan" command. - Workaround: The tagged ports are not shown but it is functioning good and also can be seen from the web. | 83702 |

Known Restrictions and Limitations

This section details the known caveats and limitations in this software version. It is recommended to avoid testing these areas, or to use the recommended workaround to test them. Features that have not yet been implemented are listed in this section. The features listed below are supported, but reflect the limitations noted below.

Table 1: Layer 2 Features

| Headline | Description | Reference |
| --- | --- | --- |
| RSTP | In RSTP, edge ports can receive BPDUs that would change the STP port mode, and still be treated as edge ports Workaround: edge port is mainly used for the case of connecting device to the switch directly. For DGS-3100, ports are rarely configured to operate in this way. | 86350 |

Table 2 Quality of Service

| Headline | Description | Reference |
| --- | --- | --- |
| Stacking Queue | It is possible to change the stacking queue method to WRR Workaround: the stacking queue has highest priority in default. The user guide can address that stacking performance will be degraded if we change the stacking queue method to WRR. | 86109 |

Table 3: Web Based Interface

| Headline | Description | Reference |
| --- | --- | --- |
| Number of sessions | It is impossible to activate the third web session Workaround: Two web sessions work properly within the stack, in standalone mode there is no problem at all. | 84364 |

Table 4: CLI Commands

| Headline | Description | Reference |
| --- | --- | --- |
| CLI: Telnet/SSH | - when get connected to telnet/ssh with user with priv-1 (user) cannot going back with priv-15 (admin) - Workaround: the CLI command, “enable admin”, does not work properly. The user should log out firstly and then log in as admin. | 83678 |
| CLI - reset | An irrelevant reboot message appears after reset the device Workaround: Reset without reboot will only reset the startup configuration while keeping the running configuration. The running configuration will be cleared if the user confirms the reboot message. | 85504 |

Table 5: Management & System Features

| Headline | Description | Reference |
| --- | --- | --- |
| Stacking Ports | It's possible to configuration Stacking port's (49-50) under LAG Workaround: the system will have the same behavior either stack ports configured as LAG or do not. | 81450 |
| SNMP | In CLI, it’s not possible to create snmp user assign with a group of v1. Workaround: Web GUI has removed the conditional check to support the creation of snmp user (v3) with a v1 group. Meanwhile, CLI does not remove the conditional check yet. | 85443 |
| Default IP Address | In a very specific scenario, the device will not reload with the default IP address, this is the scenario: Open a CLI connection See that the IP address on the device is the default one - 10.90.90.90/24 1. Execute reset and select [N] when you asked whether to reboot the device. 2. Execute reboot. 3. Once the device is reloaded execute “show switch”, see that the ip is 0.0.0.0 4. See that the configuration is empty. The CLI script: Workaround: The bug happens only with the above specific steps. | 87977 |

Revision History

Table 6: Revision History

| Revision | Date | Comment |
| --- | --- | --- |
| Revision 1.0.0.0 | May 13th, 2007 | Initial document generation |
| Revision 1.0.0.1 | May 14th, 2007 | Removed bugs 84199 & 84231 |
| Revision 1.0.0.2 | May 31st , 2007 | Updates for May 31, 2007 |
| Revision 1.0.0.3 | June 5th , 2007 | Roger Updates |
| Revision 1.0.0.4 | June 20th , 2007 | Adding known issue regarding default IP address (87997) |

This page is intentionally left blank.

Marvell Semiconductor, Inc.

700 First Avenue

Sunnyvale, CA 94089

Phone 408.222.2500

Fax 408.752.9028

www.marvell.com

| Worldwide Offices Marvell Semiconductor, Inc. 700 First Avenue Sunnyvale, CA 94089 Tel: 1.408.222.2500 Fax: 1.408.752.9028 Marvell Asia Pte, Ltd. 151 Lorong Chuan, #02-05 New Tech Park Singapore 556741 Tel: 65.6756.1600 Fax: 65.6756.7600 Marvell Japan K.K. Shinjuku Center Bldg. 50F 1-25-1, Nishi-Shinjuku, Shinjuku-ku Tokyo 163-0650 Tel: 81.(0).3.5324.0355 Fax: 81.(0).3.5324.0354 Marvell Semiconductor Israel, Ltd. 6 Hamada Street Mordot HaCarmel Industrial Park Yokneam 20692 Israel Tel: 972.4.909.1500 Fax: 972.4.909.1501 |  | Worldwide Sales Offices Western US Sales Office Marvell 700 First Avenue Sunnyvale, CA 94089 Tel: 1.408.222.2500 Fax: 1.408.752.9028 Sales Fax: 1.408.752.9029 Central US Sales Office Marvell 9600 North MoPac Drive, Suite #215 Austin, TX 78759 Tel: 1.512.343.0593 Fax: 1.512.340.9970 Eastern US/Canada Sales Office Marvell Parlee Office Park 1 Meeting House Road, Suite 1 Chelmsford, MA 01824 Tel: 978 250-0588 Fax: 978 250-0589 Europe Sales Office Marvell 3 Clifton Court Corner Hall Hemel Hempstead Hertfordshire, HP3 9XY United Kingdom Tel: 44.(0).1442.211668 Fax: 44.(0).1442.211543 Israel Sales Office Marvell Ofek Center Bldg. 2, Floor 2 Northern Industrial Zone Lod 71293 Israel Tel: 972.8.914.1300 Fax: 972.8.914.1301 |  | China Sales Office Marvell 5J, 1800 Zhong Shan West Road Shanghai, China 200233 Tel: 86.21.6440.1350 Fax: 86.21.6440.0799 Japan Sales Office Marvell Shinjuku Center Bldg. 50F 1-25-1, Nishi-Shinjuku, Shinjuku-ku Tokyo 163-0650 Tel: 81.(0).3.5324.0355 Fax: 81.(0).3.5324.0354 Taiwan Sales Office Marvell 2F, No.1, Alley 20, Lane 407, Sec. 2 Ti-Ding Blvd., Nei Hu District Taipei, Taiwan, R.O.C Tel: 886.2.8177.7071 Fax: 886.2.8752.5707 |
| --- | --- | --- | --- | --- |