- Software Release Note

(軟體發佈說明書)

- Product: DSL-2640T ADSL2+ Router

- Version: V3.02B01T01.EU-A.20091107

- Revision: 264

- Engineer by: Dongdong

(07,Nov.,2009)

This document contains confidential proprietary information and is the property of Alpha Networks Inc. The contents of this document may not be disclosed to unauthorized persons without the written consent of Alpha Networks Inc.

- Release History:

| # | Version | Date | Description | Note |
| --- | --- | --- | --- | --- |
| 1. | V3.01B01T01.EU-A.20060502 | 2006/05/02 |  |  |
| 2. | V3.02B01T01.EU-A.20060519 | 2006/05/19 |  |  |
| 3. | V3.02B01T01.EU-A.20060602 | 2006/06/02 |  |  |
| 4. | V3.02B01T01.EU-A.20060728 | 2006/07/28 |  |  |
| 5. | V3.02B01T01.EU-A.20060808 | 2006/08/08 |  |  |
| 6. | V3.02B01T01.EU-A.20061020 | 2006/10/20 |  |  |
| 7. | V3.02B01T01.EU-A.20061023 | 2006/10/23 |  |  |
| 8. | V3.02B01T01.EU-A.20061123 | 2006/11/23 |  |  |
| 9. | V3.02B01T01.EU-A.20071212 | 2007/12/12 |  |  |
| 10. | V3.02B01T01.EU-A.20080125 | 2008/01/25 |  |  |
| 11. | V3.02B01T01.EU-A.20080305 | 2008/03/05 |  |  |
| 12. | V3.02B01T01.EU-A.20080725 | 2008/07/25 |  |  |
| 13. | V3.02B01T01.EU-A.20081106 | 2008/11/6 |  |  |
|  | V3.02B01T01.EU-A.20091107 | 2009/11/7 |  |  |

Release Image Filename: DLinkEU-A_DSL-2640T_singleimage_kernel_fs_V3.02B01T01.EU-A.20091107

Bootloader Version: PSPBoot 1.3.7.15 for 4M/16M AR7WRD (Dec 1 2005 19:55:39)

Firmware Version: V3.02B01T01.EU-A.20091107

Hardware Version: V.B1

Hardware Compatibility: TI 7300EZ + 1350AZ

Software Compatibility:  NSP361+ WLAN 1350 + APDK 6.3+ Web v3.02

- ADSL Compatibility: ATM Driver 7.03.01.00  with Datapump7.03.01.00  Annex A

General Description: ADSL2+ Wireless Router with 4-port switch

- Special ADSL API Settings: N/A

- Change List:

| # | Description | Status | Note |
| --- | --- | --- | --- |
| 1. | DSL-2640T cannot enable more than 1 Filter rules | Fixed | 2009/11/7 |
| 2. |  |  |  |

- Bugs Fixed History:

| # | Description | Status | Note |
| --- | --- | --- | --- |
| 1 | After "Dynamic IP Address" connected , the Conneciotn Type of Device Info should be DHCP but not dhcpc. | Fixed | 2006/02/08 |
| 2. | When "Dynamic IP Address" connection is connected , change "Virtual circuit" to "Disable" , the IP of Default Gateway shouldn't display a blank but N/A , and IP of DNS Server shouldn't display 168.95.1.1 but N/A on the Device Info page. | Fixed | 2006/02/08 |
| 3. | When "Dynamic IP Address" connection is connected , took off the ADSL line , the IP of Default Gateway and DNS Server still displayed on the Device Info page. | Fixed | 2006/02/08 |
| 4. | When WAN Setting = Dynamic IP Address. Although WAN haven't got IP yet , the property of Status displayed "Connected" in the Device Info page. | Fixed | 2006/02/08 |
| 5. | The default value of DNS Relay Selection is "Use Auto Discovered DNS Server Only". So there's no need to have the default IP 168.95.1.1in te Preferred DNS Server column. | Fixed | 2006/02/08 |
| 6. | When DNS Relay Selection is "Use Auto Discovered DNS Server Only", the "Preferred and Alternate DNS Server" column should be blocked , it shouldn't still allow users to input. | Fixed | 2006/02/08 |
| 7. | On PPPoE/PPPoA setting page , if there's a character " was input for the User Name and Password , after pressing Apply button , " or the character after that will disappear. | Fixed | 2006/02/08 |
| 8. | Establised an IP Filter rule. ---> Press "Edit" button. ---> Configure the rule's parameters. ---> Press Apply button. ---> But the rule's configuration is still not changed. | Fixed | 2006/02/08 |
| 9. | There's no PPP page in Advanced , so PPP Configuration should be removed from Help. | Fixed | 2006/02/08 |
| 10. | The character of "You must save settings and reboot to take effect." in the Virtual Server page is different with the other pages. | Fixed | 2006/02/08 |
| 11. | In the Wireless Management page , the character of "MAC Address" in the Access List is bigger than the other letters , it's incongruent. In the Wireless Management page , the character of the entry in the Associated Stations is bigger than the other letters , it's incongruent. | Fixed | 2006/02/08 |
| 12. | In the QoS page , after clicking Help button , the connected web page is incorrect. | Fixed | 2006/02/08 |
| 13. | After setting the bandwidth of portmapping ---> Apply-->Reboot. Then the web page became so slowly and can't be opened normally. Procedure: -Factory reset. -Go to Advanced -->QoS enable the portmapping and set the bandwidth as following port 1--> pvc 0 ---->64 kbps port 2--->pvc 0----->64 kbps port 3 --->pvc 0------>64 kbps port 4----> pvc 0 ----->64 kbps -Apply--->Save and Reboot. -After rebooting the web page became so slowly and can't be opened normally. | Fixed | 2006/02/08 |
| 14. | Wireless portmapping issue: Even the OoS of wireless portmappig was fixed to PVC0 , but wireless PC2 still can ping wan PC4 via pvc 1 | Fixed | 2006/02/08 |
| 15. | DSL-G624T crash issue. | Fixed | 2006/02/08 |
| 16. | Support APDK 6.3 |  | 2006/05/19 |
| 17. | Support Web v3.02 |  | 2006/05/19 |
| 18. | Once PPPoE is connected, DHCP server IP range will be changed to WAN IP address. | Fixed | 2006/06/02 |
| 19. | Fix web browser issues for: Netscape 8.1, IE, Firefox, Mozilla, Opera | Fixed | 2006/06/02 |
| 20. | Enable the SSH port 22 in WAN side as default. |  | 2006/07/28 |
| 21. | Disable the SSH port 22 in WAN side as default. |  | 2006/08/08 |
| 22. | Fix DHCP issue |  | 2006/10/20 |
| 23. | Fix admin password and webport issue |  | 2006/10/20 |
| 24. | Add Help file no QoS explanation page. |  | 2006/10/20 |
| 25. | Fixed DDNS not working for Pacific Internet on dyndns.com in Singapore |  | 2006/10/23 |
| 26. | Fixed IP unnumbered issue. |  | 2006/11/23 |
| 27. | For the wireless fail issue, don't set NVS_TFTP_LOAD to be 1 once the wireless die ID can't be read. |  | 2007/12/12 |
| 28 | The lan ip can’t access virtual web server from the wan ip of the CPE and chage the ATM Driver and datapump to 7.03.01.00 | Fixed | 2008/01/25 |
| 29 | When access virtual server , the client ip is not the real ip of client but the ip of the CPE | Fixed | 2008/03/05 |
| 30. | Add the judge whether the virsual server rules map the same port. For example,add a user’s rule 9090 port map to 21and add a rule “FTP Server”, Now, the port 21 is be used twice. | Fixed | 2008/07/25 |
| 31 | Virtual server rule missing after DHCP lease time up Procedure | Fixed | 2008/11/6 |

- Known Issues:

| # | Description | Status | Note |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |

- Factory Default Value:

- LAN interface IP: 192.168.1.1

- WAN interfaces:

- PVC0: PPPoE LLC   VPI/VCI= 8/35

- System:

- Username/Password: admin/admin

Username1/Password1: user/user

DHCP Server: Enabled

UPnP: Disabled

DNS: Enabled

- Wireless LAN:

- SSID: DLINK_WIRELESS

- VLAN ID: 0

Priority: 0

Channel: 6

Security:  None