DVG-2001S CLI	Command

- Basic network configure:

WAN

> nwdbg ip

> nwdbg ip [manual | dhcp | PPPoE ]               ------WAN Link type

> nwdbg [ip /gw / mask ]                         ---configure static WAN IP

ex: > nwdbg ip 10.56.66.11

> nwdbg gw 10.254.254.251

> nwdbg mask 255.0.0.0

> nwdbg mac                                   ---  WAN mac

> nwdbg ip

> nwdbg mask

> nwdbg mac

> nwdbg raccesson                           --   access WEB from WAN

> nat dump                                 --   show current NAT and NAPT table

> nwdbg web port <PORT NUMBER>          - set the web port number from 1024 to 29999 and 80

PPPoE:

> pppoe mylogin <uid> <password>                    - Specify my login uid and password

ex: pppoe mylogin 84106647@hinet.net  i123456

> pppoe show                                       -- Show current pppoe settings

> pppoe connect                                     --Connect the pppoe link

DNS:

> dhcpserv dns                                   -- configure WAN first DNS

> dhcpserv dns1                                            WAN 2nd DNS

>

- SIP Configuration:

>sip set server_ip = 10.0.0.1                     --setting sip server ip

>sip set 1 phone_number = 1402                 -- setting phone number

>sip set 1 auth_username = 1111 auth_password = 1111 ---setting user and password

>nwdbg save changes                            --save setting

> sip set peertopeer mode [0/1]

> sip set cw_at [0|1]                             --  call waiting alert tone

> sip set voip caller_id [1 /0]                      -- Caller ID delivery

> sip set [1/2] display_cid [1/0]                    --- Caller ID display

ex: > sip set 1 display_cid 1              -- enable  phone port 1 CID display

> sip set 2 display_cid 1              -- enable  phone port 2 CID display

> set [tcid] default_digit_map                ---- Primus USA

> sip set mwi_enable [1/0]                     ----- Voice Mail check

> sip set [end point ID] oob [0/1/2]       0: oob disable , 1: dtmf outband  2: dtmf sip info

ex: sip set 1 oob 0                    --  set phone port 1 inband DTMF relay

sip set 2 oob 1                    --  set phone port 2  RFC2833 DTMF relay

> sip register

> sip unregister

> sip dns [show | set ] [ dns1| dns2 ]                 ---Primus USA , dns query for SIP packets

ex: sip dns set dns   168.95.1.1

sip dns set dns2  168.95.192.1

> sip set voice_mail_number

> sip status                                      -- List SIP call status.

> sip cfg                                        -- Show SIP global configurations.

> sip show                                      -- Show SIP configurations

>

- System Tool:

> show version                                -- Show f/w version

> nwdbg nmm reset                     - reset the NMM to default value,

- but keep CA, RGW,... not change.

- Then reboot the device.

> nwdbg system reboot

> nwdbg save changes                        -- save and reboot system

> nwdbg factory reset

> tftp check [on/off]                         - check the same image downloading

> nwdbg prov                              - provision function

>

- > spy sip 0

> commit                      --  Commit config to flash

> activate                       -  Activate Current Config