Topology

IPsec over dynamic IP

PC2:192.168.20.10/24

LAN1 ip:192.168.20.1/24

wan:dynamic ip address

IPsec vpn

Internet

IPsec VPN

Wan2 GW:61.218.67.121

Wan2:61.218.67.126/28                 wan1: dynamic ip address

LAN1 IP:192.168.1.1/24

PC1:192.168.1.10/24

Configure in DFL-210

(1)Go to objects->authentication objects.Create new object.

(2)go to object”dfl-210”,type name and shared secret.

The password I used “dlink”.

(3)Go to IPsec add a new object”test”.Create a new tunnel.

Local net ”lannet”

Remote Net “192.168.1.0/24”

Remote Endpoint “61.218.67.126”

(4) Go to new object “test”->”authentication”.Choose the correct Pre-Shared Key.

(5) Go to IP Rules.Add two new ip rules.”allow-go-vpn” “allow-from-vpn”

allow-go-vpn: action->allow or nat, src if->lan , src net->lannet ,

dest if->test dest net->192.168.1.0/24 service->all_services

allow-from-vpn: action->allow or nat , src if->test , src net->all-net ,

dest if->lan dest net->lannet service->all-services

Configure in DFL-800

(1) Go to address book.Add a new address “remote-net”.

(2)Change WAN2 ip address,network and default gateway.

Wan2:61.218.67.126/28 GW:61.218.67.121/28

(3) Go to interfaces -> IPsec. Add a new tunnel”test”.

Remote Endpoint does not need choose.Because the Remote endpoint is dynamic IP address.

IKE Algorithms:medium

IPsec Algorithms:medium

(4) Go to new object “test”-> “authentication”.Choose the correct Pre-Shared Key.

(5) Go to new object “test”-> “Routing”.Choose the “Dynamiclly add route”item.

(6) Go to new object “test”-> “Advanced”. Do not choose this button.

(7)Go to IP rules.Add two new rules “allow-go-vpn” and “allow-from-vpn”.

allow-go-vpn: action->allow or nat, src if->lan , src net->lannet ,

dest if->test dest net->remote-net service->all_services

allow-from-vpn: action->allow or nat , src if->test , src net->all-net ,

dest if->lan dest net->lannet service->all-services

(8)

Use pc2 ping pc1

Use CLI:ipsecstats