LAN 1:192.168.2.0/24

DFL-800

WAN1:3.3.3.2/24

IPsec Tunnel

DFL-860                                       WAN1:3.3.3.1/24

LAN:192.168.1.1/24

OSPF area 1 ; LAN 2:192.168.1.0/24

LAN: 192.168.1.2/24

L3 SWITCH

LAN: 10.10.1.1/24

OSPF area 0   BACKBONE area

LAN3:10.10.1.0/24

LAN:10.10.1.2/24

L3  SWITCH

OSPF

Add new ospf object

Click the edit this object item

Touch OSPF interface button

In the OSPF Interfaces, add the “lan” into the list.

For the LAN detail parameters as following:

Create two “Dynamic Routing Rules”: “OSPF_to_main” and “main_to_ospf” respectively.

(1)In the object of “ospf_to_main”:
Go into the Routing actions

OSPF action is mapped to main_to_ospf object

Move the main from available to selected

(2) In the object of “ospf_to_main”:

Go into the OSPF actions

DFL-860  IPsec

Create pre-shard key

Go to address book create IP object. Remote network

Go to IPsec. Add a new IPsec tunnel.

Choose the correct item.

local network to lannet.

Remote network to remote_net

Remote endpoint to 3.3.3.2/24

Encapsulation mode to tunnel

You will see IPsec tunnel route in the main.

Set up two IP rules for the tunnel:

An allow rule for outbound traffic that has the previously defined ipsec_tunnel object

As the Destination interface.The Destination network is the remote network remote_net.

An allow rule for inbound traffic that has the previously defined ipsec_tunnel object

As the Source interface.The Source network is the remote network remote_net.

Finally you can use CLI:ipsecstats to check the IPsec tunnel!!